PRESCRIBED FIRE COMPLEXITY RATING WORKSHEET

| Site: |  | Unit: |  | State: | Date: |
| --- | --- | --- | --- | --- | --- |
| Complexity Score (circle) |  |  |  |  |  |
| ## Low (44-80 pts) | Moderate (81-150 pts) |  | ## High (151-220 pts) |  |  |

Weighting Factor x Complexity Value = Total points.  Sum of Total points = Complexity Score

| Complexity Element | Weighting Factor | Complexity Value (1-5) | Total Points | # Rationale and/or Mitigation Procedures (Use for clarification of rationale and/or actions.) |
| --- | --- | --- | --- | --- |
| 1. Safety | 5 |  |  |  |
| 2. Difficulty of Containment | 5 |  |  |  |
| 3. Fuels and Fire Behavior | 5 |  |  |  |
| 4. Wildland / Urban Interface | 5 |  |  |  |
| 5. Objectives | 4 |  |  |  |
| Sub Total (Page 1) |  |  |  |  |

| Complexity Element | Weighting Factor | Complexity Value (1-5) | Total Points | # Rationale and/or Mitigation Procedures |  |
| --- | --- | --- | --- | --- | --- |
| 6. Management Organization | 4 |  |  |  |  |
| 7. Contingency Planning and Resources | 4 |  |  |  |  |
| 8. Natural, Cultural, Social Values | 3 |  |  |  |  |
| 9. Air Quality Values | 3 |  |  |  |  |
| 10. Logistics | 3 |  |  |  |  |
| 11. Tactical Operations | 2 |  |  |  |  |
| 12. Cooperator Coordination | 1 |  |  |  |  |
| Sub Total |  | Page 2 |  | Additional Comments: |  |
|  |  | Page1 |  |  |  |
| Complexity Score |  |  |  |  | Rated by: |

| Complexity Element | #### Complexity Value Evaluation Examples |  |  |
| --- | --- | --- | --- |
|  | 1 | 3 | 5 |
| 1. Safety Weighting Factor - 5 | - All safety issues have been identified and mitigated. | - A number of significant issues have been identified and some of them are difficult to address through mitigation. | - Complex safety issues exist. |
| 2. Difficulty of Containment Weighting Factor - 5 | - Low threat of escape past unit boundaries. - Probability of Ignition<50%. - Boundaries naturally defensible or firebreaks easily installed and defended. - Secondary control lines strong and easily accessed by vehicles and/or crew. | - Moderate threat of escape from unit boundaries. - 50<Probability of Ignition<70% - Moderate risk of slopover or spot fires. - Fuel type produces numerous firebrands. - Secondary control lines difficult to access or not secure. | - High threat of escape from unit boundaries. - Probability of Ignition>70%. - High risk of slopover or spot fires. - Secondary control lines non-existent or inadequate without significant resource commitment. |
| 3. Fuels and Fire Behavior Weighting Factor - 5 | - Low variability in slope & aspect. - Weather uniform and predictable. - Surface fuels (grass and/or needles) only. - No drought present or predicted within burn period. - Duff or organic soils will not ignite. | - Moderate variability in slope & aspect. - Weather variable but predictable. - Ladder fuels present and torching expected. - Fuel types/loads variable. - Dense, tall shrub or mid-seral forest communities. - Drought index indicates normal to moderate drought conditions; present expected within burn period. - Upper level of duff or organic soil will burn. | - High variability in slope & aspect. - Weather variable and difficult to predict. - Extreme fire behavior and/or stand replacement fire. - Fuel types/loads highly variable. - Altered fire regime, hazardous fuel /stand density conditions. - Drought index indicates severe drought conditions; present or expected within burn period. - Significant portions duff or organic soils will burn. |
| 4. Wildland / Urban Interface Weighting Factor - 5 | - No risk to people or property within or adjacent to fire, or values to be protected are easily mitigated. - Potential damage from escape low. | - Several values to be protected. - Mitigation through planning and/or preparations is complex. - May require some commitment of specialized resources. - Potential damage from escape moderate. | - Numerous values and/or high values to be protected. - Severe damage likely without significant commitment of specialized resources with appropriate skill levels. - Potential damage from escape high. |

| Complexity Element | Complexity Value Evaluation Examples |  |  |
| --- | --- | --- | --- |
|  | 1 | 3 | 5 |
| 5. Objectives Weighting Factor - 4 | - Maintenance objectives. - Prescriptions broad. - Easily achieved objectives. | - Restoration objectives. - Reduction of both live and dead fuels. - Moderate to substantial changes in two or more strata of vegetation. - Objectives judged to be moderately hard to achieve. - Objectives may require moderately intense fire behavior. | - Restoration objectives in altered fuel situations. - Precise treatment of fuels and multiple ecological objectives. - Major change in the structure of 2 or more vegetative strata. - Conflicts between objectives and constraints. - Requires a high intensity fire or a combination of fire intensities that are difficult to achieve. |
| 6. Management Organization Weighting Factor - 4 | - Span of control held to 2 - 3. - 6 - 12 person crew and 1 - 2 engines. | - Span of control held to 4 – 5. - Multiple resources required (engines, dozers, aerial ignition, terra torch, etc.). - 8 - 20 person crew and 1 - 3 engines. | - Span of control greater than 5 – 7. - Multiple branch, divisions or groups. - Specialized resources needed to accomplish objectives. - Organized management team required (Fire Use or Incident Management). |
| 7. Contingency Planning and Resources Weighting Factor - 4 | - Adequate contingency resources on site. | - Contingency resources limited or have more than a 15 - 30 minutes response time. | - Contingency resources limited or have more than a 30+ minutes response time. |
| 8. Natural, Cultural, and Social Values Weighting Factor - 3 | - No risk to natural, cultural, and/or social resources within or adjacent to fire, or mitigation through planning and preparations is adequate. | - Several values to be protected. - Mitigation through planning and/or preparations is complex. - May require some commitment of specialized resources. | - Numerous values and/or high values to be protected. - Severe damage likely without significant commitment of specialized resources with appropriate skill levels. |
| 9. Air Quality Values Weighting Factor - 3 | - Few smoke sensitive areas near fire. - Smoke produced for less than 1 burning period. - Air quality agencies generally require only initial notification and/or permitting. - No potential for scheduling conflicts with cooperators. | - Multiple smoke sensitive areas, but smoke impact mitigated in plan. - Smoke produced for 2-3 burning periods. - Daily burning bans are sometimes enacted during the burn season. - Infrequent consultation with air quality agencies is needed. - Low potential for scheduling conflicts with cooperators. | - Multiple smoke sensitive areas with complex mitigation actions required. - Health or visibility complaints likely. - Smoke produced for greater than 3 burning periods. - Multi-day burning bans are often enacted during the burn season. - Smoke sensitive Class I air-sheds. - Frequent consultation with air quality agencies is needed. - High potential for scheduling conflicts with cooperators. |

| Complexity Element | Complexity Value Evaluation Examples |  |  |
| --- | --- | --- | --- |
|  | 1 | 3 | 5 |
| 10. Logistics Weighting Factor - 3 | - Easy access. - Duration of fire is 1 day (holding or monitoring). | - Difficult access. - Duration of fire support between 2 and 3 days. - Logistical position assigned. - Anticipated difficulty in obtaining resources. | - No vehicle access. - Duration of support is greater than 3 days. - Multiple logistical positions assigned. - Remote camps and support necessary. |
| 11. Tactical Operations Weighting Factor - 2 | - Simple ignition patterns with only one igniter inside the unit. - Ignition complete within one burning period. - Single ignition method used. - Resources required for 1 day. - Holding requirements minimal. | - Multiple firing methods and/or sequences with two igniters inside the unit at once. - Use of specialized ignition methods (i.e. terra-torch or Premo-Mark III). - Ignition continues for two burning periods. - Resources required for 2 to 3 days. - Holding actions to direct or delay fire spread. | - Complex firing patterns highly dependent upon local conditions. - Simultaneous use of multiple firing methods and/or sequences, greater than 2 igniters inside unit. - Simultaneous ground and aerial ignition. - Use of heli-torch. - Resources required for over 3 days. - Multiple mitigation actions at variable temporal and spatial points identified. - Aerial support for mitigation actions desirable or necessary. |
| 12. Cooperator Coordination Weighting Factor - 1 | - Cooperators not involved in operations. - No concerns. | - Simple joint-jurisdiction fires. - Some competition for resources. - Some concerns. | - Complex multi-jurisdictional fires. - High competition for resources. - High concerns. |